| Stack Machines | Operand stack to replace accumulator |
| --- | --- |
|  | More intermediate results within the CPU -> less memory access required |
|  | Code density, can reuse to implement recursion |
|  | Fits naturally to equations (reverse Polish notation) |
| Reg. Machines | Small, fast local memory for intermediate results |
|  | Practical for small memories to be multiported (allows for parallelism in CPU) |
| Amdahl’s Law | speedup=entire task perf w/ enchance.entire task pery w/out enhance. |
|  | Make common case fast |
| CISC | Eliminate the semantic gap |
|  | Instruction usefulness limited or so general it is slow |
| Exceptions | Hardware (e.g. divide by 0) |
|  | Software (e.g. SWI #) |
|  | Invoke exception hander |

| Memory | SRAM: maintains store when provided with power |
| --- | --- |
|  | 4-6 transistors per bit, fast |
|  | DRAM: requires refreshing |
|  | 1 transistor per bit, fairly fast |
| Latencies | Register file: 1 cycle |
|  | L1 cache: 1-3 cycles |
|  | L2 cache: 3-9 cycles |
|  | Main memory: 10-100 cycles |
|  | Note that DRAM can do burst reads and writes: 2-8 cycles |
|  | Hard disk: 106 cycles! |
| Locality | Temporal: access recently used memory again soon |
|  | Spatial: access close to recently used memory soon |
| Cache Line | Group of around 4 neighbouring words stored |
| Fully Associative | Store any word anywhere in the cache, lookup by address |
|  | Huge overhead involved! |
| Direct Mapping | Use part of the address to map onto a cache line |
|  | Compare tag / valid flag in line to determine if it really caches the address |
|  | Behaves badly when using data from overlapping addrs. |
| Set Associative | As direct mapping, but with a set of cache lines at each location |
| Victim | As direct mapped but with a one line buffer to store the last line overwritten |
| Cache Line Replacement | Only applied to set/fully associative caches, clearly |
|  | LRU: requires more info. |
|  | NLU: pass the “potato” on if a cache line is accessed |
|  | Random: simple, works well |
| Writes | If data is already in the cache then write over it |
|  | If not in cache then either fetch on write / write around |
| Write Through | Data is written to both cache and lower level memory |
|  | Common on multiprocessor systems for cache coherency |
|  | Use bus snooping on SMP (does not scale > 2 CPUs) |
| Write Back | Data is written to cache only |
|  | Written to main memory upon replacement, can use dirty bit to prevent this when it is unnecessary |
| Write Buffer | Avoid CPU stalling by buffering writes |
|  | Also perform write merging: can take advantage of burst |
| Virtual and Physically Addressed | V + P addresses only differ in upper bits, if cache is no bigger than a page then can use lower bits of V to access the cache without conflict |
|  | Otherwise use P address for cache or access with V and use P (concurrently found) to compare tags in cache |
|  | Still have problems with aliasing of P addresses in V |
| TLB | Cache recent translations |
|  | Fully associative cache |
|  | TLB miss must be looked up |
|  | Potential control hazard |

| Pipelining | Increases frequency and latency (due to latch time) |
| --- | --- |
|  | Best to make pipeline stages of similar length |
| Exceptions | Imprecise: deep pipeline |
|  | Precise: when you want OS to be able to restart it |
| Instruction Replays | Only find out about cache misses in next stage! |
|  | This means that the pipeline has advanced too far: must replay instructions to give cache hardware more time |
|  | While waiting for cache, refill the pipeline with instructions from the I-cache |
| Control Hazard | Caused by branching and not clearing the pipeline |
|  | Could document behaviour and use branch delay slots |
| Data Hazard | Caused by not taking account of results yet to be written back that is still in other pipeline stages |
|  | Use feedforward/bypass |
|  | Sometimes have to stall pipeline if later instructions depend on e.g. memory fetches: can introduce bubbles, so minimise this |
|  | Could document behaviour and use load delay slots |
|  | Otherwise have hardware detect it (e.g. scoreboarding) |

| Parallel | Synchronous communication |
| --- | --- |
|  | Timing assumption can be violated by skew: doesn’t work for high f or long s |
| Serial | Asynchronous transmission |
|  | Use 8B/10B coding to guarantee the run length (allows clock recovery), DC balance (allows AC coupling) |

| Control Flow | Concurrency simulated via interrupts / scheduler |
| --- | --- |
|  | Has to throw away register file, disrupt caching/pipeline |
|  | Load operations cause stall |
| Data Flow | Model dependency graph in CPU, execute it concurrently |
|  | Inherently concurrent and latency tolerant |
|  | Easy to take advantage SMP |
|  | Too much concurrency, so assignment is a problem |
|  | Makes I/O difficult |
|  | Ineffective use of very local storage (e.g. register, stack) |
|  | Scheduling policies simple |
| More information may be needed, check what the exam questions ask on this  |  |