| Layering | Law of Demeter |
| --- | --- |
|  | Peer entities communicate |
| Channels | Symbols, delay, fidelity, cost, security, ordering, connectivity.. |
| Definition | Noise: systematic/random |
|  | Attenuation: radiation loss/spatial dispersion/absorption |
|  | Baud rate: symbol transmit rate |
|  | Modulation: systematic alteration of carrier by information signal |
|  | Baseband: not carrier modulated |

| Synchronisation | Asynchronous: divide transmission into frames. Oscillators in rx/tx are close in frequency, sync. clocks w/ start/stop bits |
| --- | --- |
|  | Synchronous: transmission continuous, continually sync frequency of rx (PLL, Manchester coding) |

| Modulation Of Analog Info. | Amplitude: A(mx(t)+1)cos(2πfct) |
| --- | --- |
|  | Frequency: Acos(2π(fct+fΔx(t)t)) |
|  | Phase: Acos(2π(fct+φΔx(t))) |
| Modulation Of Digital Info. | Amplitude Shift Keying: A(mx(t)+1)cos(2πfct) |
|  | Frequency Shift Keying: Acos(2π(fct+fΔx(t)t)) |
|  | Phase Shift Keying: Acos(2π(fct+φΔx(t))) |
| Phase Shift Keying Finale | Filter to remove high frequency shifts |
|  | Also have QPSK (4 levels) |
|  | Can have > 1 bit/Hz |
|  | Coherence: sync. phase change to carrier freq. |

| Coding | Information  Symbols |
| --- | --- |
|  | One entities symbols are another’s information.. |
|  | Digitisation (has one time quantisation noise) |
|  | Sampling (sample at 2B) |
| FEC | Introduce redundant info. |
| Block Codes | Divide info into fixed size messages (length m), messages encoded into codewords (length k) |
|  | Have m < k for FEC |
|  | Called a (m, k) code, code rate = m / k |
|  | Distance between codewords: number of bits in which they differ |
|  | Decode by looking at received word and pick the closest valid codeword |
|  | If minimum distance d then can detect d – 1 errors or correct (d – 1) / 2 errors |
| Compression | Perfect, imperfect, stable |
|  | Exploit domain e.g. JPEG quality table, MPEG moving blocks |
| Encryption | Symmetric secret keys allows authentication (w/ challenge), integrity (w/ encrypted signature), confidentiality (clearly!) |

| Multiplexing | Produce many higher layer channels from lower chan. |
| --- | --- |
|  | Policy: determine who gets a part of lower layer |
| Sharing Media | Trivial routing, requires trust due to fragility |
|  | Non-shared scales better |
| Orthogonal Multiplexing | FDM, TDM (discriminate by content/schedule), STDM (periodic slots, constant delay/bandwidth) ATDM (use packets) |
| ATDM | Packets on shared media |
|  | Appropriate if demands from higher layer variable |
| ATDM Contention | Statistical multiplexing, not fixed delay/capacity |
|  | Have policy, may be a distributed one |
|  | Random access: check for collision for 2*channel delay. If collision time < packet time then stop transmitting else retry. Simple, fault tolerant, but access time not bounded |
|  | Token passing: has problems with maintaining single token all the time |
|  | Reservations: useful with large delays, still need reservation channel |
|  | Slotted: channel has slots like STDM but “cells” are contended for. Reservation system lets you run a synchronous service by periodic allocation |
| Non Orthogonal Multiplexing | Use functions which are “nearly” orthogonal |
| CDMA | Each channel has a unique pseudo-random sequence |
|  | Cycle through sequence transmit it XOR with data |
|  | Receiver XORs same sequence with received data, looks for correlations to get sequence sync |
|  | Good for mobiles: single frequency, codes don’t change during handoff |

| Ethernet | Shared using CSMA/CD |
| --- | --- |
|  | Collision window is twice the cable length, same as minimum packet size |
|  | Retransmit lightens up given a busy network by backing off longer |
|  | Routers isolate collisions |
|  | Throughput depends on distribution of requests |
| Token Ring | Tokens have priority |
|  | High throughput, low latency guaranteed |
|  | Monitor station ensures only one token: stations contend to be the monitor |
| Slotted Ring | To prevent circulating full slots a monitor sets/ checks the monitor bit in each circulating frame |
|  | Good latency if source delete, pass empty slot on |

| Error Control | Error detect + retransmit |
| --- | --- |
| ARQ | Transmit information in frames, receiver acknowledges frames with correct CRC. Transmitter resends unacknowledged frames after a timeout / gets a potential NACK |
|  | Time from beginning of one data frame to next is: 2τd+pb (frame size p) |
|  | This shows a dependency on latency being low |
| Continuous ARQ | Have multiple frames in transmit at once |
|  | If window is big enough the link can be kept full |
|  | Upon missing frame, either go back or do selective retransmission |

| Flow Control | Balance long term information rates |
| --- | --- |
| X-On X-Off | Receiver needs to receive 2 channel delays of information after stop |
| Sliding Window | Combine error/flow control |
|  | Receiver tells transmitter what frames are received, how far ahead it allows |
|  | Change window with buffer availability |

| Definitions | Name: denotes something |
| --- | --- |
|  | Address: denotes where something is |
|  | Route: tells you how to get there |
|  | Name lookup: binding a name to an address |
|  | Routing: bind an address to a route |
| Address Spaces | Flat (moved addresses without modification) |
|  | Hierarchical (divided to aid the routing process) |
| Routing | Static/dynamic |
|  | Central/distributed |
|  | e.g. ARP via broadcast |
|  | Repeater (regenerates signal), bridge (forwards between two MACs), routers (knows about structure of addresses, uses it to route) |
|  | Flood (robust, tries shortest path), random, shortest path (but requires knowledge of whole network, inconsistencies cause e.g. loops) |
|  | Source routing: sender decides route, embedded in packet. Can be loose |
| When To Route | Per packet (robust, adapts to network, no ordering) |
|  | Per connection (less computation, ordering, lets resources be allocated) |
|  | Virtual circuits have connection setup and state but don’t have fixed resource allocations |
|  | Datagrams have neither: they can be routed solely based on their contents |

| The Internet | IP addresses have network and host parts |
| --- | --- |
|  | Router checks for network being one of its networks |
|  | If not, consult routing table of (network address, next router) pairs |
|  | Also have default route |
|  | Routers exchange info. |
| TCP | Streaming, connection oriented, reliable, full duplex, flow controlled |
|  | Determine capacity by loss |
| UDP | Datagrams only |
| Standards | ITU (UN: modems, framing etc), IEEE (LANs), ISO (models), IETF (applications! RFCs etc.) |

| Circuit Switching | You and a receiver share a dedicated channel |
| --- | --- |
|  | Hardware is dedicated to you, so typically latency and capacity are fixed |
| Packet Switching | Multiple senders and receivers |
|  | Entities can transmit at different rates |
|  | Stateless routing |
|  | Send to many receivers without circuit setup overhead |
|  | Intelligent local routing |
|  | Graceful degradation |