Logic Definitions

| Literal | Boolean variable |
| --- | --- |
| Minterm | Conjunction of all variables complemented or otherwise |
| DNF | Disjunction of all minterms |
| Maxterm | Disjunction of all variables complemented or otherwise |
| CNF | Conjunction of all maxterms |
| Cover | Term covers a minterm if that minterm is part of the tem |
| Implicant | A product term T is an implicant of a formula G iif G is true for all input combinations that make T true |
| Prime Implicant | A product term T is a prime implicant of a formula G iif T is an implicant of G and no proper subterm of T is an implicant of G |
| Essential Term | A prime implicant which covers a minterm that no other prime implicant covers |
| Covering Set | A minimum set of prime implicants which includes all essential terms |
| Static Hazard | Output glitches but returns to the original level |
| Dynamic Hazard | Output changes state but glitches during the change |

Circuits

| Fast Carry Adder | gn=an.b,pn=an+b cn+1=gn+pn.c⇒cn+2=(gn+1+pn+1.gn)+(pn+1.pn).c |  |
| --- | --- | --- |
| RS Latch |  |  |
| D Latch |  |  |
| D Flip Flop | !D !D When Clock Goes Low D When Clock Goes Low !D, Changes To 0 If D Changes When Clock Is Low -VE Edge Triggered |  |
|  | Or alternatively two clocked RS latches in series. |  |
| Obscure Sync. Flip Flops | J; K; Q’. 0; 0; Q. 0; 1; 0. 1; 0; 1. 1; 1; !Q. | T; Q’. 0; Q. 1; !Q. |

State Machines

| Finite State Machine | Deterministic circuit producing outputs dependent on its internal state and external inputs |
| --- | --- |
| States | Set of internal memorized values, shown as circles on diagrams |
| Inputs | External stimulus to the machine, shown as arcs on diagrams |
| Outputs | Results from a FSM, associated with a state (Moore) or a function of the state and inputs (Mealy) |
| Start Up | Reset or self starting |
| State Encoding | Shift, sequential, sliding, one hot |

Building Gates From Transistors

| FETs | nMOS: pMOS: |
| --- | --- |
| Tristate Buffer |  |

Programmable Logic Devices

| ROM | Implements table lookup O(m*2n) |
| --- | --- |
| PAL | Selectively remove connections in NOT, AND and OR planes, scale with number of terms not states |