| Logic Models | Four valued logic: “don’t know” and “high impedance” |
| --- | --- |
|  | Can add further to simulate charge holding elements etc |
| Delays | Pure delay: delayed by a time constant |
|  | Inertial delay: capacitive delay modelled |
| Simulation | Simple model: scan netlist and update everything |
|  | Delta simulation: only notify gates when on input change |
| Behavioural Verilog | Models combinatorial logic to construct a state machine |
| Structural Verilog | Hierarchic netlist form sometimes generated by a Verilog compiler |
| Assignment | Continuous (assign x = …) |
|  | Blocking (x = …) |
|  | Non-blocking (x <= …) |

| QM Minim. | Find all prime implicants: - Produce a truth table and extract minterms - Compare pairs of terms that differ by 1 bit to produce a new term where the difference becomes x - Tick those terms that have been covered by the new term - Repeat with the new set of terms until no more terms can be produced - Unticked terms are prime implicants |
| --- | --- |
|  | Select smallest set of prime implicants to cover function: - Prepare chart showing how implicants cover the minterms - Select essential prime implicants (i.e. those that cover unique minterms) - Select other prime implicants as necessary to cover |
|  | Don’t cares important for minimisation so you should use “x” a lot in Verilog |
| Other Minim. | Wire minimisation: mainly done by place + route, but synthesis may help by duplicating logic and hinting |
|  | State minimisation: remove duplicate/unreachable states |
|  | State assignment: can be done automatically |

| Faults | Logical: stuck at, stuck open, bridging faults |
| --- | --- |
|  | Parametric: low/high voltage, gate and path delay |
|  | Equivalence: remove test for least significant fault |
|  | Dominance: if even test for 1 detects 2 then 1 dominates 2 |
|  | Undetectable: no test exists |
|  | Redundant: undetectable and doesn’t affect operation |
| Testability | Controllability |
|  | Observability |
| D-Algorithm | ~D: 0 in good, 1 in bad |
|  | D: 1 in good, 0 in bad |
|  | - Assign each checkpoint D or ~D - Propagate D forward - From a fault detecting primary output work backwards to determine inputs to make the fault visible |
| Scan Paths | Make all the D latches scannable (solve state problem) with multiplexers |
|  | Aids ATPG, latches have sliced circuit into combinatorial logic |